| Test Pharmaceuticals Ltd. govind nagar ANALYTICAL METHOD VALIDATION PROTOCOL |  |
| --- | --- |

| NAME OF PRODUCT | Test Product |
| --- | --- |
| LABEL CLAIM | EACH VIAL CONTAIN: Test Ingredient 100mg |
| PROTOCOL NO. AMV/P/TEST/001 | PAGE 1 OF 20 |

ANALYTICAL METHOD VALIDATION PROTOCOL FOR ASSAY
OF
Test Product

| Test Pharmaceuticals Ltd. govind nagar ANALYTICAL METHOD VALIDATION PROTOCOL |  |
| --- | --- |

| NAME OF PRODUCT | Test Product |
| --- | --- |
| LABEL CLAIM | EACH VIAL CONTAIN: Test Ingredient 100mg |
| PROTOCOL NO. AMV/P/TEST/001 | PAGE 2 OF 20 |

CONTENTS

1. Approval	3

2. Overview	4

2.1 Objective	4

2.2 Scope	4

2.3 Responsibility	4

3. Details of Instruments/ Equipment	4

3.1 List of Equipment and Instrument Used	4

3.2 List of Glass or Other Materials	4

3.3 List of Reagents and Prepared Solutions:	5

3.4 Working Standard Details:	5

4. References: ICH Q2 (R2)	5

5. Methodology	5

6. Validation Parameter	7

6.1 System Precision	8

6.2 Specificity	9

6.3 Method Precision	10

6.4 Intermediate precision (Ruggedness):	12

6.5 Linearity and Range	15

6.6 Accuracy/Recovery:	17

6.7 Robustness	19

7. Validation Report	20

| Test Pharmaceuticals Ltd. govind nagar ANALYTICAL METHOD VALIDATION PROTOCOL |  |
| --- | --- |

| NAME OF PRODUCT | Test Product |
| --- | --- |
| LABEL CLAIM | EACH VIAL CONTAIN: Test Ingredient 100mg |
| PROTOCOL NO. AMV/P/TEST/001 | PAGE 3 OF 20 |

1. Approval

This is a specific protocol for analytical method validation for Assay of Test Product by HPLC.
This protocol has been approved by the following:

|  | Name | Department | Signature | Date |
| --- | --- | --- | --- | --- |
| Prepared By | shivam | Analyst Q.C | [Signature] | 11/11/2025 |
| Checked By | annay | Asst. Manager Q.C | [Signature] | 11/11/2025 |
| Approved By | ayush | Manager Q.C | [Signature] | 11/11/2025 |

| Test Pharmaceuticals Ltd. govind nagar ANALYTICAL METHOD VALIDATION PROTOCOL |  |
| --- | --- |

| NAME OF PRODUCT | Test Product |
| --- | --- |
| LABEL CLAIM | EACH VIAL CONTAIN: Test Ingredient 100mg |
| PROTOCOL NO. AMV/P/TEST/001 | PAGE 4 OF 20 |

2. Overview

2.1 Objective

To establish the methodology for the analytical method validation for Assay of Test Product by HPLC.

2.2 Scope

This Validation is applicable for the determination of Assay of Test Product by HPLC.

2.3 Responsibility

• Executive QC.
- To prepare the analytical method validation protocol and to carry out the analytical work In accordance with this protocol.
- To carry out all operations in accordance with GLP and relative Standard Operating Procedures (SOPs).
- To record all observations.
• Assistant Manager QC
- To check the protocol and report.
• Head Quality
Head Quality will approve the Protocol and Report.

3. Details of Instruments/ Equipment

3.1 List of Equipment and Instrument Used:

The following apparatus / equipment shall be used for validation studies:
a) glass (Code: 244c812) - ghjk

3.2 List of Glass or Other Materials

The following glass or other materials shall be used for Validation studies:
a) glass - 10ml

3.3 List of Reagents and Prepared Solutions:

The following reagents and chemicals shall be used for Validation studies:

| Reagent Name | Batch Number | Expiry Date |
| --- | --- | --- |
| hplc | 6531 | 18/11/2025 |

3.4 Working Standard Details:

| Name of Working Standard | Potency |
| --- | --- |
| Test Ingredient | 99.5% |

4. References: ICH Q2 (R2)

5. Methodology

Product	Methodology Code
Test Product	TEST/METHOD/001
Chromatographic system:

Mode	: LC
Detector	: UV 280 nm
Column	: 4.5 mm X 25 cm; 5 µm L1
Injection volume	: 
Autosampler	: 10°
Column	: 50°
Flow rate	: 
Solution A: Dissolve 2.6 ml of Tetrabutylammonium hydroxide solution (40% in water) and 2.8 gm of disodium hydrogen phosphate in 1000 ml of water. Adjust with phosphoric acid to a pH of 7.8. Mobile Phase: Methanol and Solution A (150:850)

Standard solution: Accurately weigh and transfer about 100.0 of Test Ingredient working standard into a 100.0 volumetric flask. Add about 50ml of diluent and sonicate to dissolve. Make up the volume with diluent and mix well.

Sample solution: Weigh and powder 36.95 tablets. Transfer accurately weighed powder equivalent to 100mg of Test Ingredient into a 100.0 volumetric flask. Add about 50ml of diluent and sonicate for 15 minutes with intermittent shaking. Make up the volume with diluent and mix well. Filter the solution through 0.45µ PVDF syringe filter, discarding first few ml of filtrate.

Procedure: Inject  of blank, standard solution (six replicate injections) and sample solution (in duplicate) into the chromatograph. Record the chromatograms and measure the peak responses. Calculate the content of Test Ingredient per tablet.

| Test Pharmaceuticals Ltd. govind nagar ANALYTICAL METHOD VALIDATION PROTOCOL |  |
| --- | --- |

| NAME OF PRODUCT | Test Product |
| --- | --- |
| LABEL CLAIM | EACH VIAL CONTAIN: Test Ingredient 100mg |
| PROTOCOL NO. AMV/P/TEST/001 | PAGE 6 OF 20 |

Procedure: Inject 10µl injection of each solution as given below:

| Sample ID | No. of injection |
| --- | --- |
| Blank | 01 |
| Standard solution | 06 |
| Sample solution | 03 |

System suitability:

| Sr. No. | System suitability parameter | Acceptance criteria |
| --- | --- | --- |
| 1 | Tailing factor | NMT 2.0 |
| 2 | %RSD of area in the standard solution replicates. | NMT 2.0% |

Calculation:

Analysis Samples: Standard solution and Sample Solution

Calculate the percentage of the labeled amount of Test Ingredient (C4H9N3O2) in the portion of Test Product taken:

Result = (rU/rS) × (CS/CU) × (Mr1/Mr2) × 100

Where:
rU = peak response of Test Ingredient from the Sample solution
rS = peak response of Test Ingredient from the Standard solution
CS = concentration of Test Ingredient working standard in the Standard solution (mg/ml)
CU = nominal concentration of Test Ingredient in the Sample solution (mg/mL)
Mr1 = molecular weight of Test Ingredient, 131.13
Mr2 = molecular weight of Test Ingredient salt form, 131.13

Acceptance Criteria: 95.0% - 105.0%

Limit: It contains not less than 95.0 percent and not more than 105.0 percent of the labeled amount of Test Ingredient.

| Test Pharmaceuticals Ltd. govind nagar ANALYTICAL METHOD VALIDATION PROTOCOL |  |
| --- | --- |

| NAME OF PRODUCT | Test Product |
| --- | --- |
| LABEL CLAIM | EACH VIAL CONTAIN: Test Ingredient 100mg |
| PROTOCOL NO. AMV/P/TEST/001 | PAGE 7 OF 20 |

6. Validation Parameter:

The HPLC method is evaluated for following validation parameters:

| Sr.No. | Validation Parameters |
| --- | --- |
| 1 | System Suitability |
| 2 | Specificity |
| 3 | System Precision |
| 4 | Method Precision |
| 5 | Accuracy/Recovery |

| Test Pharmaceuticals Ltd. govind nagar ANALYTICAL METHOD VALIDATION PROTOCOL |  |
| --- | --- |

| NAME OF PRODUCT | Test Product |
| --- | --- |
| LABEL CLAIM | EACH VIAL CONTAIN: Test Ingredient 100mg |
| PROTOCOL NO. AMV/P/TEST/001 | PAGE 8 OF 20 |

6.1 System Precision:

The system precision of method is demonstrated by injecting the Blank/diluent, and Standard solution. For preparation of diluent, blank solution (diluent), Standard solution and chromatographic conditions refer to section 6.0 i.e. analytical methods.

Acceptance criteria:

✓ System Suitability should meet the requirement.
✓ The relative Standard deviation of the replicate injections obtained from six replicates of Standard solution should be not more than 2.0%.
✓ Tailing factor obtain from Standard solution is NMT 2.0

| Test Pharmaceuticals Ltd. govind nagar ANALYTICAL METHOD VALIDATION PROTOCOL |  |
| --- | --- |

| NAME OF PRODUCT | Test Product |
| --- | --- |
| LABEL CLAIM | EACH VIAL CONTAIN: Test Ingredient 100mg |
| PROTOCOL NO. AMV/P/TEST/001 | PAGE 9 OF 20 |

6.2 Specificity:

To ensure the interference from blank and placebo solution those is likely to be present at the peak due to in Standard and sample solution.

Standard solution: Taken 12.0 mg of Test Ingredient RS/WS and transfer it into 100 ml volumetric flask add water to dissolve the content sonicate if necessary, make up volume with water up to 100ml.
Placebo Solution: Take placebo solution equivalent to sample (except API) in 100ml volumetric flask. Pipette 2.5ml of resulting solution and transfer to 100ml volumetric flask, make volume upto 100 ml with water.
Sample solution: Take 4 vials of sample and reconstitute with water shake well to dissolve and transfer the content to 100ml volumetric flask. Rinse the same vials 2 to 3 times with water and transfer to the same volumetric flask and make up volume 100ml with water. Further pipette 2.5ml of above solution and transfer to 50ml volumetric flask, make volume upto 50 ml with water.

Procedure: Inject 10µl of the above solutions in HPLC and record the chromatogram and check peak purity.

| Sample | Number of Injections |
| --- | --- |
| Blank Solution | 1 |
| Standard Solution | 1 |
| Sample Solution | 1 |
| Standard Solution Bkt. | 1 |

Acceptance Criteria

✓ System Suitability should meet the requirement.
✓ No significant interfering peak should appear in the blank chromatogram at the retention time of the main peak. Peak Purity should pass.

| Test Pharmaceuticals Ltd. govind nagar ANALYTICAL METHOD VALIDATION PROTOCOL |  |
| --- | --- |

| NAME OF PRODUCT | Test Product |
| --- | --- |
| LABEL CLAIM | EACH VIAL CONTAIN: Test Ingredient 100mg |
| PROTOCOL NO. AMV/P/TEST/001 | PAGE 10 OF 20 |

## 6.3 Method Precision

Method precision will be evaluated using six sample preparations.

Acceptance criteria: %RSD ≤ 2.0%

| Test Pharmaceuticals Ltd. govind nagar ANALYTICAL METHOD VALIDATION PROTOCOL |  |
| --- | --- |

| NAME OF PRODUCT | Test Product |
| --- | --- |
| LABEL CLAIM | EACH VIAL CONTAIN: Test Ingredient 100mg |
| PROTOCOL NO. AMV/P/TEST/001 | PAGE 12 OF 20 |

## 6.4 Intermediate Precision

Intermediate precision will be evaluated by different analysts on different days.

Acceptance criteria: %RSD ≤ 2.0%

| Test Pharmaceuticals Ltd. govind nagar ANALYTICAL METHOD VALIDATION PROTOCOL |  |
| --- | --- |

| NAME OF PRODUCT | Test Product |
| --- | --- |
| LABEL CLAIM | EACH VIAL CONTAIN: Test Ingredient 100mg |
| PROTOCOL NO. AMV/P/TEST/001 | PAGE 15 OF 20 |

## 6.5 Linearity and Range

Linearity will be evaluated from 80% to 120% of target concentration.

Acceptance criteria: r ≥ 0.999

| Test Pharmaceuticals Ltd. govind nagar ANALYTICAL METHOD VALIDATION PROTOCOL |  |
| --- | --- |

| NAME OF PRODUCT | Test Product |
| --- | --- |
| LABEL CLAIM | EACH VIAL CONTAIN: Test Ingredient 100mg |
| PROTOCOL NO. AMV/P/TEST/001 | PAGE 17 OF 20 |

## 6.6 Accuracy/Recovery

Recovery will be evaluated at 80%, 100%, and 120% levels.

Acceptance criteria: 98.0% - 102.0%

| Test Pharmaceuticals Ltd. govind nagar ANALYTICAL METHOD VALIDATION PROTOCOL |  |
| --- | --- |

| NAME OF PRODUCT | Test Product |
| --- | --- |
| LABEL CLAIM | EACH VIAL CONTAIN: Test Ingredient 100mg |
| PROTOCOL NO. AMV/P/TEST/001 | PAGE 19 OF 20 |

## 6.7 Robustness

Robustness will be evaluated for critical method parameters.

Acceptance criteria: No significant impact on results

| Test Pharmaceuticals Ltd. govind nagar ANALYTICAL METHOD VALIDATION PROTOCOL |  |
| --- | --- |

| NAME OF PRODUCT | Test Product |
| --- | --- |
| LABEL CLAIM | EACH VIAL CONTAIN: Test Ingredient 100mg |
| PROTOCOL NO. AMV/P/TEST/001 | PAGE 20 OF 20 |

# 7. Validation Report

A comprehensive validation report will be generated summarizing all results.